CODE

/*************************************************************

Project: ESP8266 Blynk Wi-Fi Robot (Motors + Pump + Servo + Ultrasonic safety)

Hardware: NodeMCU + L298N + Relay + Servo + HC-SR04

Author: Ish

Notes: TRIG = D3, ECHO = D2. Uses V8 for distance (cm) and V10 for status messages.

*************************************************************/

// --- Blynk Template Info ---

#define BLYNK_TEMPLATE_ID "TMPL3U2rGpfY7"

#define BLYNK_TEMPLATE_NAME "WiFi Robot"

#define BLYNK_AUTH_TOKEN "Y-O90VmQklqC9eoBadBmNUn-bP7hWER8"

// --- Libraries ---

#include <ESP8266WiFi.h>

#include <BlynkSimpleEsp8266.h>

#include <Servo.h>

// --- Wi-Fi Credentials ---

char ssid[] = "KaranIphone";

char pass[] = "karan123";

// --- Motor Driver Pins (L298N) ---

#define IN1 D5

#define IN2 D6

#define IN3 D7

#define IN4 D8

// --- Relay Pin for Pump ---

#define RELAY_PIN D4

// --- Servo Pin ---

#define SERVO_PIN D1  // NodeMCU D1 (GPIO5)

// --- Ultrasonic Pins ---

#define TRIG_PIN D3   // NodeMCU D3 (GPIO0)

#define ECHO_PIN D2   // NodeMCU D2 (GPIO4)

// --- Ultrasonic conversion ---

#define SOUND_VELOCITY 0.034  // cm per microsecond

// --- Virtual Pins ---

#define VFORWARD   V1

#define VBACKWARD  V2

#define VLEFT      V3

#define VRIGHT     V4

#define VSTOP      V5

#define VPUMP      V6

#define VSERVO     V7

#define VDIST_CM   V8   // Distance (cm)

#define VSTATUS    V10  // Status messages

// --- Safety threshold ---

const float STOP_LEFT_THRESH = 40.0;  // Left blocked ≤ 40 cm

BlynkTimer timer;

Servo servo;

bool leftBlocked = false;  // left-block flag

// --- Movement helper functions ---

void moveForward()  { digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW);  digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);  }

void moveBackward() { digitalWrite(IN1, LOW);  digitalWrite(IN2, HIGH); digitalWrite(IN3, LOW);  digitalWrite(IN4, HIGH); }

void turnLeft()     { digitalWrite(IN1, LOW);  digitalWrite(IN2, HIGH); digitalWrite(IN3, HIGH); digitalWrite(IN4, LOW);  }

void turnRight()    { digitalWrite(IN1, HIGH); digitalWrite(IN2, LOW);  digitalWrite(IN3, LOW);  digitalWrite(IN4, HIGH); }

void stopMotors()   { digitalWrite(IN1, LOW);  digitalWrite(IN2, LOW);  digitalWrite(IN3, LOW);  digitalWrite(IN4, LOW);  }

// --- Blynk movement handlers ---

BLYNK_WRITE(VFORWARD)  { if (param.asInt()) moveForward(); else stopMotors(); }

BLYNK_WRITE(VBACKWARD) { if (param.asInt()) moveBackward(); else stopMotors(); }

BLYNK_WRITE(VLEFT)  {

if (param.asInt()) {

if (!leftBlocked) turnLeft();

else {

stopMotors();

Blynk.virtualWrite(VSTATUS, "⛔ Left blocked (obstacle ≤ 40 cm)");

}

} else stopMotors();

}

BLYNK_WRITE(VRIGHT) { if (param.asInt()) turnRight(); else stopMotors(); }

BLYNK_WRITE(VSTOP)  { if (param.asInt()) stopMotors(); }

// --- Pump (Relay) ---

BLYNK_WRITE(VPUMP) {

int pumpState = param.asInt();

digitalWrite(RELAY_PIN, pumpState ? HIGH : LOW);

Serial.println(pumpState ? "Pump ON" : "Pump OFF");

Blynk.virtualWrite(VSTATUS, pumpState ? "Pump ON" : "Pump OFF");

}

// --- Servo ---

BLYNK_WRITE(VSERVO) {

int pos = constrain(param.asInt(), 0, 180);

servo.write(pos);

Serial.print("Servo moved to: ");

Serial.println(pos);

Blynk.virtualWrite(VSTATUS, String("Servo: ") + pos);

}

// --- Ultrasonic measurement + left-stop logic ---

void sendDistanceToBlynk() {

// Trigger pulse

digitalWrite(TRIG_PIN, LOW);

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// Measure echo

unsigned long duration = pulseIn(ECHO_PIN, HIGH, 30000);

float distanceCm = -1.0;

if (duration == 0) {

distanceCm = -1.0;

Serial.println("Distance: out of range");

Blynk.virtualWrite(VDIST_CM, 0);

Blynk.virtualWrite(VSTATUS, "Distance: out of range");

return;

}

distanceCm = (duration * SOUND_VELOCITY) / 2.0;

Serial.print("Distance (cm): ");

Serial.println(distanceCm, 2);

// Send to Blynk

Blynk.virtualWrite(VDIST_CM, distanceCm);

Blynk.virtualWrite(VSTATUS, String("Distance: ") + String(distanceCm, 2) + " cm");

// --- Left stop logic ---

if (distanceCm <= STOP_LEFT_THRESH) {

if (!leftBlocked) {

leftBlocked = true;

stopMotors();

Serial.println("🚨 Forward BLOCKED: Obstacle ≤ 40 cm");

Blynk.virtualWrite(VSTATUS, "🚨 Forward BLOCKED: Obstacle ≤ 40 cm");

}

} else {

if (leftBlocked) {

leftBlocked = false;

Serial.println("✅ Safe: Forward motion allowed (>40 cm)");

Blynk.virtualWrite(VSTATUS, "✅ Safe: Forward motion allowed (>40 cm)");

}

}

}

// --- Setup ---

void setup() {

Serial.setDebugOutput(false);

Serial.begin(115200);

delay(100);

Serial.println("Connecting to Wi-Fi...");

WiFi.begin(ssid, pass);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println();

Serial.print("Wi-Fi connected! IP: ");

Serial.println(WiFi.localIP());

Serial.println("Connecting to Blynk...");

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

// Initialize pins

pinMode(IN1, OUTPUT);

pinMode(IN2, OUTPUT);

pinMode(IN3, OUTPUT);

pinMode(IN4, OUTPUT);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, LOW);

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

servo.attach(SERVO_PIN);

servo.write(90);

stopMotors();

Serial.println("Setup complete. Ready!");

// Ultrasonic update every 1 sec

timer.setInterval(1000L, sendDistanceToBlynk);

}

// --- Loop ---

void loop() {

Blynk.run();

timer.run();

}